# 1. Offline-First Cold-Chain Integrity Monitoring

### Recommended project title

Design of an Offline-First Edge-IoT System for Cold-Chain Risk Detection and Equipment-Failure Prediction

## Why it ranks first

This is the easiest project to complete reliably because the monitored variables—temperature, humidity, door status and power availability—can be reproduced and tested safely in a laboratory. It is derived directly from SIH MA1216, which calls for monitoring risks affecting temperature-sensitive products and generating timely alerts.

## Minimum viable prototype

The device should:

- Continuously monitor refrigerator and product-zone temperature.

- Detect prolonged door opening and power failure.

- Store readings locally when the internet is unavailable.

- Produce buzzer, display and mobile alerts.

- Synchronise stored records when connectivity returns.

- Estimate whether temperature is rising toward an unsafe condition.

## Suggested hardware

| Component | Purpose |
| --- | --- |
| ESP32 | Main controller and Wi-Fi/BLE communication |
| Two DS18B20 probes | Air and product-simulation temperature |
| SHT31/DHT22 | Humidity measurement |
| Magnetic reed switch | Refrigerator-door monitoring |
| DS3231 RTC | Accurate timestamping during network failure |
| MicroSD module | Offline data storage |
| Buzzer and OLED | Local warning and status |
| Battery/UPS module | Operation during power failure |
| Optional GSM/LoRa | Remote alerts without Wi-Fi |

Planning budget: ₹4,000–₹7,000, excluding the refrigerator and laptop.

## Ten-week action plan

| Weeks | Activity |
| --- | --- |
| 1 | Define temperature limits, system architecture and test conditions |
| 2–3 | Connect and calibrate sensors; implement local display and alarms |
| 4 | Add RTC-based logging and microSD storage |
| 5 | Develop Flask or MQTT dashboard |
| 6 | Implement store-and-forward offline synchronisation |
| 7 | Add temperature-rise prediction and alert prioritisation |
| 8 | Conduct door-open, power-cut and communication-loss tests |
| 9 | Run continuous 48–72-hour validation |
| 10 | Final enclosure, documentation and demonstration |

## Evaluation metrics

- Temperature-measurement error

- Alert-generation delay

- Battery-backup duration

- Percentage of records recovered after reconnection

- Number of false alarms

- Prediction time before a threshold excursion

## Societal impact

The system could support small pharmacies, vaccine transport boxes, rural clinics, laboratories, dairy sellers and food-storage operators. The prototype should be positioned as an early-warning and record-keeping system, not as a certified medical device.

# 2. Low-Cost Predictive and Condition-Based Maintenance

### Recommended project title

Edge-Intelligent Multi-Sensor Condition Monitoring System for Motors and Pumps in MSMEs

## Why it ranks second

This project closely matches your Communication and Networking background because it integrates sensors, embedded signal processing, MQTT communication, edge analytics and dashboards. The uploaded SIH1583 statement specifically targets equipment-health observation, telemetry, anomaly detection and maintenance decision support.

## Minimum viable prototype

Monitor one induction motor, exhaust fan or water pump and detect:

- Excessive vibration

- Overheating

- Abnormal current consumption

- Mechanical looseness

- Rotor imbalance

- Overload conditions

## Suggested hardware

| Component | Purpose |
| --- | --- |
| ESP32 or STM32 | Data acquisition and feature extraction |
| ADXL345/industrial accelerometer | Vibration measurement |
| Current transformer or isolated current sensor | Motor-current monitoring |
| DS18B20/MLX90614 | Surface-temperature measurement |
| Hall sensor | Rotational-speed measurement |
| Raspberry Pi 4 | Gateway, database and analytics |
| Wi-Fi or LoRa module | Machine-to-gateway communication |
| Buzzer/status LEDs | Local condition indication |

Planning budget: ₹6,000–₹12,000, excluding the motor and test bench.

## Twelve-week action plan

| Weeks | Activity |
| --- | --- |
| 1 | Select machine and identify safe, controllable fault conditions |
| 2–3 | Install vibration, current and temperature sensors |
| 4 | Establish sampling, filtering and timestamp synchronisation |
| 5 | Calculate RMS, peak, crest factor, kurtosis and temperature rise |
| 6 | Add FFT-based frequency analysis |
| 7 | Develop MQTT transmission and Raspberry Pi dashboard |
| 8 | Record normal machine-operation data |
| 9 | Introduce controlled imbalance, looseness or overload safely |
| 10 | Train threshold-based or lightweight anomaly-detection model |
| 11 | Evaluate false alarms, latency and fault separability |
| 12 | Package the sensor node and complete report/demo |

## Evaluation metrics

- Fault-detection accuracy

- False-alarm rate

- Detection latency

- Packet-delivery ratio

- Maximum communication range

- Difference between healthy and faulty vibration signatures

- Edge-processing time

## Societal and industrial impact

A successful low-cost unit could help small workshops monitor pumps, fans and motors without purchasing expensive industrial systems. Potential benefits include reduced unexpected downtime, improved maintenance planning and longer machine life.

## Main feasibility condition

You need safe access to a rotating machine. Faults should be simulated through mild imbalance, controlled load or loose mounting rather than damaging the equipment.

# 3. Low-Cost Livestock Health and Behaviour Monitoring

### Recommended project title

Offline LoRa-Based Livestock Behaviour and Abnormality Alert System for Small Farmers

## Why it ranks third

This project offers the strongest direct benefit to rural communities and has entrepreneurship potential. The hardware is manageable, but collecting reliable animal-behaviour data requires more time than cold-chain or machine monitoring. It should therefore be selected only when access to cattle, goats or a farm is available.

## Minimum viable prototype

A collar-mounted unit should monitor:

- General movement and inactivity

- Standing, lying and walking patterns

- Possible restlessness

- Collar-zone temperature

- Prolonged reduction in activity

- Communication loss and battery status

The first prototype should provide abnormal-behaviour alerts, not attempt to diagnose a specific disease.

## Suggested hardware

| Component | Purpose |
| --- | --- |
| ESP32-C3 or nRF52 | Low-power embedded controller |
| MPU6050/ICM-20948 | Movement and posture sensing |
| Digital temperature sensor | Collar-zone temperature trend |
| LoRa module | Long-range, low-power communication |
| RTC | Time-based activity logging |
| Rechargeable battery | Wearable power source |
| Waterproof enclosure and collar | Field deployment |
| Raspberry Pi/ESP32 gateway | Data reception and dashboard |
| Optional microphone | Rumination or vocalisation research |

Planning budget: ₹5,000–₹10,000 for one collar and gateway.

## Fourteen-week action plan

| Weeks | Activity |
| --- | --- |
| 1 | Select animal type and define only observable behaviours |
| 2–3 | Assemble low-power collar prototype |
| 4 | Develop accelerometer and temperature logging |
| 5 | Implement LoRa communication and local storage |
| 6 | Optimise sleep modes and battery consumption |
| 7–8 | Collect labelled motion data through controlled movement tests |
| 9–10 | Conduct limited supervised trials on one or more animals |
| 11 | Extract activity level, posture and inactivity-duration features |
| 12 | Implement rule-based or TinyML abnormality detection |
| 13 | Measure false alerts, range and battery endurance |
| 14 | Improve enclosure, dashboard and documentation |

## Evaluation metrics

- Activity-classification accuracy

- False abnormality-alert rate

- Battery life

- LoRa communication range

- Packet-loss rate

- Wearability and enclosure stability

- Time required to identify prolonged inactivity

## Societal impact

The project could help small farmers observe animals remotely, particularly during night hours or when animals graze away from the house. Its realistic benefit is earlier attention to unusual behaviour and reduced manual monitoring effort. Increased farmer income is a possible long-term outcome, but it should not be claimed without field evidence.

## Main feasibility condition

Farmer cooperation, ethical handling and labelled field data are essential. Without farm access, this project becomes difficult to validate convincingly.